Scouts Australia
Victorian Branch

Heany Park Scout Group

Makeham Ave, off Golding Ave

Rowville Vic 3178

A.B.N. 39 662 387 026

Postal Address
PO Box 2789
Rowville Vic 3178

Internet: 	www.heanyparkscouts.com.au

Email: 	gl.heany-park@vicscouts.asn.au

Heany Park Scout Group Climbing Wall Instructions

General Requirements:-

- Climbing Wall (CW) to be set up on firm, even and level surface.

- Maximum of 2 climbers at any one time.

- Minimum of 2 supervising personnel while the CW is in use.

- Personnel involved in assembly, supervision & disassembly must be aware of the requirements of the CW.

- Ensure the CW is assembled using all components including horizontal beams, bolts (both horizontal & vertical), rear ballast plate & ballast

- Minimum of 750kg ballast or equally rated tie-down straps to an anchor point(s) must be in position before use. A vehicle can be parked over the ballast plate.

- Do use CW when raining, stormy or in wet conditions. Cover CW with tarp when raining & overnight if set up outdoors.

- Cease use of CW where wind gusts may exceed 20km. Consider disassembling wall or adding additional tie downs to secure CW in the event of high winds.

- Climber’s feet should not exceed 1800mm from ground level.

- Safety mats must be in position before use.

IN FELLOW SHIP WE LIVE

General Information: -

- Assembled CW is 2.4m high x 6m wide

- Climbing boards & steel uprights are numbered & are position specific

- The steel uprights & rear ballast plates are heavy & require a minimum of 2 adults to handle.

- The climbing boards are 2.4m long & 1.2m wide & require a minimum of 2 people to handle.

- It is recommended that a minimum of 4 adults assessable & disassemble the CW.

Components & Tools

• 6 x Steel Uprights

• 5 x Climbing Boards with threaded holes for climbing holds

• 3 x Horizontal 2-piece Beam Sets (an upper, mid & lower set)

• Assorted coloured climbing holds

• Allen key fasteners to secure Climbing Holds to Climbing Boards – Note there are long, med & short fasteners to suits the various thicknesses of the coloured Climbing Holds

• 2 x “T” piece Allen Key tools

• Long bolts inserted vertically to secure Horizontal Beams to Steel Uprights

• Short bolts with pin holes to attach Climbing Boards to Horizontal Beams

• Hammer to assist with inserting & removing bolts pins – do not use excessive forced. Ensure area is level to assist with alignment of holes.

Assembly (Reverse order for Disassembly)

- Position Steel Uprights in numerical order. (#1 is on LHS when facing front of CW)

- Use personnel to hold uprights in position & take care that they don’t fall over

- Place the mid-level Horizontal Beam into position & insert vertical pin (long bolts)

- Continue placing Steel Uprights & Horizontal Beams till assembled. Ensure all vertical pins have been inserted & tapped in with hammer as necessary.

- Install Steel Ballast Plate (2-piece) at rear of Steel Uprights – holes in plates to secure in position via upright bolts on Steel Uprights

- Place Climbing Boards in numerical order as shown on rear of boards.

- Secure Climbing Boards with Horizontal Pins (short bolts). Fit over-sized nut & secure with steel nail into hole in bolt. Ensure Horizontal Pins are installed & secured.

- Install Climbing Holds using Allen Key fasteners – Note 3 lengths & must use correct size fastener to match thickness of Climbing Holds. Secure fasteners firmly but do not over tighten

- Place Safety Mats in position & ensure are snug with no gaps

IN FELLOW SHIP WE LIVE

IN FELLOW SHIP WE LIVE

IN FELLOW SHIP WE LIVE